High-performance door ASSA ABLOY RR300 Plus

Description:

Flexible High-performance door, opening in vertical direction for interior use in wind-protected areas.

Self-supporting side frames made from Aluminium anodized or stainless steel.

Door curtain made from transparent PVC with fabric reinforcement stripes, ultramarine (similar to RAL 5002) or red (similar to RAL 3002).

Wind protection: through wind tabs on the door curtain.

Wind resistance: class 1 (EN 12424).

Bottom profile: Alunimium anodized.

Drive unit: Hollow shaft gear motor 0,65 / 0,75 kW.

Protection: IP 54.

Control system: MCS-R relais with integrated display.

Supply voltage: 3/(N)PE 400V; 50-60 Hz.

Safety devices:

- electrical safety contact edge,

- door line photocell in side frame,

- drop down protection by integrated balancing system.

Manual activation: by releasing the brake at ground level. Depending on width/height ratio the door opens up to max. 2.0 m, minimum 500 mm.

Opening / closing speed: up to max. 2.2/0.9 m/s.

Size min.: (DW x DH): 1000 x 5000 mm

Size max.: (DW x DH): 1000 x 5000 mm

Typ: ASSA ABLOY RR300 Plus

Options

Surface / Material:

- Side frame made of stainless steel

- Bottom profile made of stainless steel V2A

- Bottom profile flexible “Flex-Edge”

Cover:

- Top roll cover made from Aluminium galvanized

- Top roll cover made from steel powder coated

- Top roll cover made from stainless steel 00/300

- Motor cover made from

- Motor cover made from steel powder coated

- Moteor cover made from stainless steel 150

Motor cover required for doors DH < 2500 mm

Door curtain (alternatively):

- PVC with coloured fabric reinforcement strips

- Orange (siminlar RAL 2011)

- Grey (siminlar RAL 7035)

- Yellow (siminlar RAL 1021)

- Green (siminlar RAL 6018) 
- Beige (siminlar RAL 1014)

- White (siminlar RAL 9016)

- PVC with colored fabric reinforcement strips silicone-free
- Orange (siminlar RAL 2011)
- Grey (siminlar RAL 7037)

- Yellow (siminlar RAL 1021)

- White (siminlar RAL 9010)

- Black (siminlar RAL 9005)

- RollTex® Plus/RollTex®  Original 
- Blue (siminlar RAL 5002)
- Orange (siminlar RAL 2009)  
- Grey (siminlar RAL 7001)

- NomaTex®   
- Blue (siminlar RAL 5005)
- Red (siminlar RAL 3000)
- Orange (siminlar RAL 2009)
- Yellow (siminlar RAL 1023)

- Top-Door curtain 1,3 mm (max. 3500 x 3500 mm)
- Blue (siminlar RAL5010)
- Red (siminlar RAL 3002)
- Grey (siminlar RAL 7035)

- Window (BxH) 500x700 mm (only for door curtain RollTex® and NomaTex®)

- Window (BxH) 700x500 mm (only for door curtain RollTex® and NomaTex®)

- Vision panel horizontal for door curtain NomaTex®

- Door curtain NomaTex® flame retardant without vision panel or window – color grey
(similar RAL 7045)

Control system (alternatively):

- Frequency converter control system MCC, based on vector control including boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service

- (open/close) up to max. 2.2/0.9 m/s

- Supply voltage 3L(N)PE/380/400/415/440/480V;50/60Hz

- With transformer: 1L(N)/PE:220/230V; 50/60 Hz

- Protection IP55

- Motor power 0,75 kW

- ACS 50 contactor control system
- Open/close up to max. 1.2/1.2 m/s
- Supply voltage 3L/(N)PE: 230/380/400/415V;50/60Hz
- With transformator: 3/(N)PE: 440/480/500V;50/60 Hz 
- Protection IP55
- Motor power 0,75 KW

- MCS-FC Frequency converter controll system with integrated display 
- Open/close 1.7/0.8 m/s
- Supply voltage 1/(N)PE: 230V 50/60 Hz 
- Protection IP54
- Motor power 0,75 kW

Bottom profile:

- Model in stainless steel

- Model with Flex Edge (flexible bottom profile without aluminium extrusion and coiled cable)

Safety features:

- Integrated light curtein inside side frame, not availably in combination crash system

- Crash-System at curtain / bottom profil

- Curtain tension

- Emergency egress (crank handle or chain drive)

- Self opening in event of power outage

- Bottom profile with extended flexible seal

Miscellaneous:

- Bottom profiel with extended flexibel seal

- Gear heating

- Actuator

Option: Escape and rescue routes ASSA ABLOY RR300 Plus F+R:

Size min.: (DW x DH): 1000 x 4000 mm

Size max.: (DW x DH): 2300 x 4000 mm

Control unit:

Solely frequency converter control MCC with extension and special software (configuration).

Drive unit: hollow shaft gear motor with operating current brake

Wind resistance: class 1 (EN 12424) up to DH = 3500 mm, class 0 (EN 12424 at DH > 3500 mm

Emergency opening: In case of power failure the door opens automatically to a maximum height of 2.0 m (depending on the size).

A radar motion detector for the detection of people is included in delivery.

Key switches for the deactivation of the escape and rescue function is optionally available.