# ASSA ABLOY DS6060VC Van Cushion Shelter

## Product description:

DS6060VS Van cushion shelter

- 4-sided dock shelter with cushions of variable depths which are fastened around the perimeter of the docking opening

- The size and shape of the shelter is the ideal solution for vans like e.g. Mercedes Sprinter, Fiat Ducato, Iveco Daily, and VW Crafter with the typical curved shape and rear hinged doors that you can flip around completely to 270 degrees

- Cushions made of foam material covered by one layer quality polyester

- Straight shape of the bottom and top cushion

- Conical shape of the side cushions

- Signs on bottom and top cushion Yellow/black warning tape

- Warning sign on bottom cushion "do not step on"

## Technical specification 4-sided cushion dock shelter:

- Foam density 18 kg m³

- Back plate Water resistant MDF board 19 mm thick, suitable for humid conditions

- Fabric cushions One layer high quality polyester

  - Coating Front and back

  - Thickness 0,5 mm

  - Total weight Approx. 680 g/m²

  - Flammability DIN 75200

  - Tensile strength DIN 53345Longitudinal strength Approx. 2700 N/5cm

  - Transverse strength Approx. 2300 N/5 cm

  - Tear elongation DIN 53363

  - Longitudinal strength Approx. 300 N

  - Transverse strength Approx. 250 N

## Sizes 4-sided cushion dock shelter:

  - Nominal width: 	2.000 mm

(Daylight width bottom 1.400 mm, top 1200 mm)

  - Nominal height:	2.350 mm

(Daylight height 1.800 mm)

  - Nominal depth bottom 250 mm

  - Nominal depth top  400 mm

  - Side cushion bottom 300 mm

  - Side cushion top 400 mm

  - Top cushion 300 mm

  - Bottom cushion 250 mm

## Technical specification separate roof (optional):

- Frame of aluminum profiles and galvanized steel profiles

- Fabric roof and side sections, ONE layer high quality polyester

  - Coating Front and back

  - Thickness 0,5 mm

  - Total weight Approx. 680 g/m²

  - Flammability DIN 75200

  - Tensile strength DIN 53345Longitudinal strength Approx. 2700 N/5cm

  - Transverse strength Approx. 2300 N/5 cm

  - Tear elongation DIN 53363

  - Longitudinal strength Approx. 300 N

  - Transverse strength Approx. 250 N

- Fabric top curtain, DOUBLE layer high quality polyester

  - Coating Front and back

  - Thickness 3,0 mm

  - Total weight Approx. 3400 g/m²

  - Flammability DIN 75200

  - Tensile strength DIN 53345

  - Longitudinal strength Approx. 7000 N/5cm

  - Transverse strength Approx. 5000 N/5 cm

  - Tear elongation DIN 53363

  - Longitudinal strength Approx. 750 N

  - Transverse strength Approx. 900 N

## Sizes separate roof (optional):

  - Nominal width: 	2.200 mm

  - Nominal height:	800 mm

  - Nominal depth:	600 mm

## Sizes complete dock shelter with separate roof (optional):

  - Nominal width: 		2.000 / 2.200 mm

  - Nominal height:		3.050 mm

  - Installation height:		3.450 mm

  - Nominal depth:		250, 400 / 600 mm

## Options:

- Nominal width: 2100 mm
(daylight width bottom 1500 mm, top 1300 mm)

- Top cushion with camera recess

- Top cushion with 200 mm top flap

- Side cushions with wear shields 1.200 x 200 mm, extra layer, 3 mm thick

- Version without bottom cushion

- Installation type cushion shelter to steel frame inside door opening

- Separate roof nominal width 2.200 mm (for dock shelter width 2.000 mm)

- Separate roof nominal width 2.300 mm (for dock shelter width 2.100 mm)

Options wall fixing:

- Concrete wall

- Insulated wall

- Light concrete wall

- Cavity wall

- Steel frame