# ASSA ABLOY DL6010S Swingdock

## Product description:

### Stationary dock leveler with swing lip with:

- Integrated frame with wall anchors (T-frames)

- Electro-hydraulic operation with two lifting cylinders

- Hydraulic cylinder swinging out of swing lip in highest platform position

- Automatic, free floating position

- One-button control unit with mains switch, including interlocking of leveler/door

- Hold-to-run button to position lip on the truck bed and to put the leveler back in parking position

- Hydraulic emergency stop in any operating position

- Automatic mechanical locking in neutral position

- Lateral torsion flexibility

- Side plates (toe guards) with warning signs both above and below dock level

- Internal wiring, plug connection

- Power supply by others

- Control Unit Protection Class IP 54

- The dock leveler meets standard demands of most loading operations and fully complies with rules and regulations of the European Standard EN 1398

## Dimensions:

| Dimensions |  | Vertical working range |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LL 400 |  | LL 500 |  |
| NL OL GL LH |  | A | B | A | B |
| 2000 | NL+330 NL+190 600 | 260 | 280 | - | - |
|  | NL+330 NL+190 700 | 290 | 330 | 180 | 360 |
| 2250 | NL+330 NL+190 600 | 290 | 275 | - | - |
|  | NL+330 NL+190 700 | 300 | 345 | 180 | 380 |
| 2500 | NL+330 NL+190 600 | 310 | 270 | - | - |
|  | NL+330 NL+190 700 | 390 | 340 | 270 | 360 |
| 2770 | NL+330 NL+190 600 | 335 | 270 | - | - |
|  | NL+330 NL+190 700 | 400 | 340 | 290 | 360 |
| 3000 | NL+330 NL+190 600 | 340 | 265 | - | - |
|  | NL+330 NL+190 700 | 400 | 335 | 280 | 290 |
| 3500 NL+330 NL+190 800 |  | 495 | 335 | 390 | 345 |
| 4000 NL+330 NL+190 900 |  | 540 | 335 | 430 | 345 |
| 4500 NL+330 NL+190 900 |  | 590 | 335 | 480 | 345 |

Nominal width (NW) 1750, 2000, 2200. Load bearing capacity: 60 kN

## Surface:

- Surface painted in standard color RAL 5010 / RAL 9005 , thickness 80 µm

## Lip bearing:

- Reinforced steel tear plate swing lip

- 400 mm swing lip with 5° bend

- 40 mm bevelled lip

## Hydraulics:

- Standard oil for temperature range -20° to +60° Celsius

## Control unit:

- Voltage 400V, cable harness length 7m

- Display of error messages

- Service interval display

- Readout function / Memory function

## Options:

### Frame

- T-200 Frame (standard frame with a height of 200mm)

- W-Frame (welded to pre-cast frame)

- F-Frame (flat bar frame welded to existing frame or for replacement)

- B-Frame (box frame, for positioning dock leveler and subsequent setting in concrete)

- P-Frame (pit frame with support from under (made by others), up to NL=3000mm)

### Surface

- Surface painted in RAL 6005 / RAL 3002, thickness 80µm

- Surface painted with coat thickness 160µm

- Surface hot-dipped galvanised

### Hydraulics

- Low temperature oil for temperature range -30° to +60° Celsius

- Bio-oil (environmental protection)

### Lip bearing

- Swing lip 500mm (only for LH700 and larger)

- Bevelled lip 100 mm

- Straight swing lip

- Tapered swing lip

### Energy & Ergonomics

- EPDM gap sealing

- Slip protection and noise reduction coating

### Multifunctional docking control for direct control of dock leveler, dock shelter and door with just one control unit

- Operating dock leveler and door

- Operating dock leveler dock shelter and door

- Operating dock leveler and dock shelter

### Control unit

- Impulse auto button to put the leveler back in parking position

- Parking position detection

- Voltage 400V

- Cable harness extension

- External EMERGENCY STOP button