2021-03

ASSA ABLOY

DL6120ST Stepdock

Tender Specification Text

# ASSA ABLOY DL6120ST Stepdock

Product description

Stationary dock leveler with telescopic lip with

- Integrated frame with wall anchors (T-frames)

- T-frame with step 900 x 200 mm

- Specifically for the sequence “dock first, open rear truck doors later”

- Lip length 1000 mm

- ISO loading system, leveler placed behind the door

- Electro-hydraulic operation with two lifting cylinders

- One hydraulic cylinder for infinitely-adjustable extension of steel lip

- Automatic, free floating position

- Three-button switch with standard main switch, incl. contact to lock door / leveler

- Hold-to-run button to lift platform

- Hold-to-run button to position the lip on the truck bed

- Impulse auto button to put leveler back in parking position

- Hydraulic emergency stop in any operating position

- Automatic mechanical locking in neutral position

- Lateral torsion flexibility

- Side plates (toe guards) with warning signs both above and below dock level

- Internal wiring, plug connection

- Power supply by others

- Control Unit Protection Class IP54

- The dock leveler meets standard demands of most loading operations and fully complies with rules and regulations of the European Standard EN 1398

Dimensions

| NL | LH | LE 1000 mm |  |
| --- | --- | --- | --- |
|  |  | A | B |
| 2500 | 700 | 300 | 450 |
| 3000 | 750 | 420 | 410 |

Nominal width (NW) 2000, 2200 mm for all sizes.

Resting position

- Horizontal

Surface

- Surface painted in standard color RAL 5010, thickness 80 µm

Lip bearing

- Reinforced steel tear plate swing lip

- 100 mm bevelled lip

Hydraulics

- Standard oil for temperature range -20°C - +60°C

Control unit

- Voltage 400V 3-phase, 230V 3-phase, cable harness length 7m

- Display of error messages

- Service interval display

- About function / Memory function

- Impulse auto button to put the leveler back in parking position

Options

Resting position

- In slope

Surface

- Surface painted in RAL 9005, RAL 6005, RAL 3002, thickness 80 µm

- Surface painted with coat thickness 160 µm

- Surface hot-dipped galvanised

Hydraulics

- Low temperature oil for temperature range -30°C - +60°C

- Bio-oil (environmental protection)

Bridge bearing

- Steel lip with front sill of 100mm

- Lip in aluminium

- Lip with oblique section 100mm

- Lip with retracting tongues in steel, 1 per side (140mm)

Energy & Ergonomics

- EPDM seal (ONLY resting position in slope)

- Slip protection and noise reduction coating

- Front cover curtain PVC (ONLY resting position horizontal)

Multifunctional docking control for direct control of dock leveler, dock shelter and door with just one control unit

- Operating dock leveler and door

- Operating dock leveler dock shelter and door

- Operating dock leveler and dock shelter

Control unit

- Request parking position

- Voltage 400V 3-phase, 230V 3-phase

- Cable harness extension

- External EMERGENCY STOP button

Application description ISO loading system

- In closed position, the dock leveler is very well sealed off from below, preventing cold or hot air access.

- Only available with lip length 1000 mm

- With insulated panels mounted into the pit underneath the dock leveler.

The thickness of the panels is available in 42 or 82 mm.

Options of the ISO loading system

- Panels for ISO loading system in thickness 42 mm

- Panels for ISO loading system in thickness 82 mm