2020-01

ASSA ABLOY

DL6120T Teledock

Tender Specification Text

# ASSA ABLOY DL6120T Teledock

Product description

Stationary dock leveler with telescopic lip with

- Integrated frame with wall anchors (T-frames)

- Electro-hydraulic operation with two lifting cylinders

- One hydraulic cylinder for infinitely-adjustable extension of steel lip

- Automatic, free floating position

- Three-button switch with standard main switch, incl. contact to lock door / leveler

- Hold-to-run button to lift platform

- Hold-to-run button to position the lip on the truck bed

- Impulse auto button to put leveler back in parking position

- Hydraulic emergency stop in any operating position

- Automatic mechanical locking in neutral position

- Lateral torsion flexibility

- Side plates (toe guards) with warning signs both above and below dock level

- Internal wiring, plug connection

- Power supply by others

- Control Unit Protection Class IP54

- The dock leveler meets standard demands of most loading operations and fully complies with rules and regulations of the European Standard EN 1398

Dimensions

| Dimensions | Vertical working range 60 kN |
| --- | --- |

|  | LE 500 mm | LE 1000 mm | LE 345 mm |
| --- | --- | --- | --- |

| NL | LH | A | B | A | B | A | B |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2000 | 600 | 310 | 310 | 380 | 370 | 270 | 280 |
|  | 700 | 340 | 400 | 420 | 470 | 370 | 360 |
| 2500 | 600 | 420 | 260 | 500 | 290 | 380 | 225 |
|  | 700 | 430 | 370 | 510 | 420 | 400 | 340 |
| 3000 | 600 | 370 | 230 | 430 | 250 | 350 | 210 |
|  | 800 | 450 | 400 | 520 | 440 | 420 | 360 |
| 3500 | 800 | 520 | 400 | 600 | 440 | 490 | 360 |
| 4000 | 900 | 590 | 400 | 660 | 440 | 560 | 380 |
| 4500 | 900 | 570 | 410 | 640 | 440 | 550 | 380 |

Nominal width (NW) 1750, 2000, 2200, 2250mm for all sizes.

Surface

- Surface painted in standard colour RAL 5010, thickness 80 µm

Lip bearing

- Reinforced steel tear plate swing lip

- 500 mm telescopic lip

- 100 mm bevelled lip

Hydraulics

- Standard oil for temperature range -20°C - +60°C

Control unit

- Voltage 400V 3-phase, 230V 3-phase, cable harness length 7m

- Display of error messages

- Service interval display

- About function / Memory function

- Impulse auto button to put the leveler back in parking position

Options

Surface

- Surface painted in RAL 9005, RAL 6005, RAL 3002, thickness 80 µm

- Surface painted with coat thickness 160 µm

- Surface hot-dipped galvanised

Frame

- W-Frame (welded to pre-cast frame)

- F-Frame (flat bar frame welded to existing frame or for replacement)

- B-Frame (box frame, for positioning dock leveler and subsequent setting in concrete)

- P-Frame (pit frame with support from under (made by others), up to NL=3000mm)

Hydraulics

- Low temperature oil for temperature range -30°C - +60°C

- Bio-oil (environmental protection)

Bridge bearing

- Steel lip with front sill of 100mm

- Lip in aluminium

- Lip with oblique section 100mm

- Lip with retracting tongues in steel, 1 per side (140mm)

- Lip 1000 mm

Energy & Ergonomics

- EPDM seal

- Slip protection and noise reduction coating

- Front cover curtain PVC (up to nominal length of 3000 mm)

Multifunctional docking control for direct control of dock leveler, dock shelter and door with just one control unit

- Operating dock leveler and door

- Operating dock leveler dock shelter and door

- Operating dock leveler and dock shelter

Control unit

- Request parking position

- Voltage 400V 3-phase, 230V 3-phase

- Cable harness extension

- External EMERGENCY STOP button

Application description ISO loading system

- In closed position, the dock leveler is very well sealed off from below, preventing cold or hot air access.

- Only available with lip length 1000 mm

- With insulated panels mounted into the pit underneath the dock leveler

The thickness of the panels is available in 42 or 82 mm.

Options of the ISO loading system (only with lip length 1000 mm)

- T-Frame for ISO loading system

- Panels for ISO loading system in thickness 42 mm

- Panels for ISO loading system in thickness 82 mm