# ASSA ABLOY DS6060F Curtain dock shelter fix

## Product description:

DS6060F Curtain dock shelter fix

- Structure consists of 3 extruded sections which are fitted into the sides of an extension

- Easily replaceable curtains

- Curtain quality

  - Top curtain /side curtains:

    - Double-layered high quality polyester, coated on both sides

    - Total weight approx. 3400g/m²

    - Flammability (DIN 75200)

    - Tensile strength:

    - Tear elongation:

DIN 53354

longitudinal strength: approx. 7000N/5cm transverse strength: approx. 5000N/5cm DIN 53363

longitudinal strength: approx. 750N transverse strength: approx. 900N

- Top curtain with a slit in the main wear area to minimize contact pressure from the truck

- Standard variants

  - Top curtain:

- Top curtain 1000 mm

- Splitted top curtain 1000 mm (4 slits in the wear area)

- Top curtain 1200 mm

- Splitted top curtain 1200 mm (4 slits in the wear area)

  - Nominal depth:

- Nominal depth 600 mm

- Nominal depth 900 mm

  - Side curtain:

- Side curtain 600 mm (Nominal width 3250 mm)

- Side curtain 700 mm (Nominal width 3450 mm)

  - Colour:

- black

- blue (RAL 5010, side and front curtains)

  - Colour of parking guides

- white

- yellow (sim. to RAL 1023)

  - Wall fixings:

- Concrete

- Sandwich

- ISO panel wall

- Light concrete

- Loadhouse

## Dimensions:

mm	Nominal height

mm	Nominal width

Options:

- Digit on top curtain (Height 300 mm)

- Roof part with integrated rain channel

- Reinforced rubber flaps in the side curtain

- Notching on both sides

- Corner seals to prevent draft in lower area ESR, ES 600, ES 900