# ASSA ABLOY DS6070B inflatable dock shelter bags

## Product description:

DS6070B inflatable dock shelter bags

- Provides optimal weather protection for temperature-controlled warehouses during loading

- The dock shelter is inflated around the docked vehicle

- Suitable for all kinds and sizes of vehicle

- The extension consists of walls and roof in ISO panel 40 mm with front plate

- Side sections with steel guards as protection

- Yellow guide lines in polyethylene

- Surface treatment hot-dipped galvanised and painted with polyester paint to comply with Corrosion protection class 3 requirements.

- Top and side bags with fabric panels of quality polyester

- Curtains flame resistant, colourfast and tear resistant

- Strong blower for straight movement of top and side bags

- Strong blower for constant pressure on the truck during loading

- Incl. blower control, fully wired

- Curtain quality of bags

  - Top

    - Backing fabric of quality polyester, coated on both sides

    - Total weight approx. 500 g/m²; 0.5 mm thick

  - Sides:

    - Backing fabric of quality polyester, coated on both sides

    - Total weight approx. 600 g/m²; 0.6 mm thick

  - Sides in contact area with the truck:

    - Backing fabric of quality polyester, coated on both sides

    - Total weight approx. 2500 g/m²; 2 mm thick

## Dimensions:

| Nominal width | 3600mm |  |
| --- | --- | --- |
| Nominal height | _mm | (3755/4055/4355/ 4555mm) |
| Top bag extension | 1000mm |  |
| Side bag extension | 650mm |  |
| Nominal depth | 770mm |  |
| Panel colour | Exterior RAL 9010 | Interior RAL 7032 |
| Colour curtains/bags | Black |  |

Options:

- Nominal depth 1020 mm

- Top bag extension 1300 mm

- Door number, on front panel

- Special colour Exterior panel, RAL buyer's choice

- Special colour Exterior front panel, RAL buyer's choice

## Wall fixings

- Concrete/light concrete

- Steel/rails

- Insulating wall up to 120 mm