2022-03

ASSA ABLOY

HS9120GHY high speed door

Tender Specification Text

# ASSA ABLOY HS9120GHY high speed door

Product description

- The HS9120GHY high speed door is designed for interior applications.

- Door frame in Stainless steel AISI 304.

- Gravity drive system.

- Unique side column technology provides excellent seal on all 4 sides thereby reducing heat loss.

- The speed of opening is up to up to 1,2 m/s (optional 2,0 m/s) depending on size of door.

- The speed of closing is 0,5 m/s.

- The door design withstands overpressure or underpressure in specific areas.

- Safety light grids in side columns.

- Dimensions: Min: 1000 mm x 2000 mm, Max: 4000 mm x 4000 mm

- Wind load resistance: (EN 12424) Class 1 (300 Pa (N/m²)). Maximum wind load while closing 50 Pa (N/m²)

- Resistance to water penetration: (EN 12425) Class 2 (50 Pa (N/m²)) 20 minutes water spray

- Air permeability (EN 12426) Class 1 (24 m³/m²/h at 50 Pa)

- Thermal transmittance (EN 12428) 6,02 W/(m²K)

- Temperature working range: 5 °C to +40 °C

- Door curtain 900 g/m² colored PVC, material thickness: 0,8 mm

- Flexible door curtain without ridged parts.

- Flexible soft bottom beam.

- Break-away and self reset function.

- The door curtain is available in 11 standard colors and translucent.

- Various window options available, material PVC, material thickness 2.0 mm.

- The motor with frequency converter guarantees smooth and reliable operation.

- Soft start/stop significantly prolongs motor lifespan.

- Operator 220V - 240V 1-phase 0.75kW 16A

- Operator protection: IP65

- Control unit with display, impulse Up and Down buttons, an emergency stop mushroom button and a mechanical main switch.

- Control unit protection: IP65

- Lifetime expectations: 1.000.000 cycles

Options

Window, vision panel and insect screen options

- small window 640x580 mm PVC

- vision panel 400mm PVC

- vision panel 800mm PVC

- small insect screen 640x580 mm PES

- insect screen panel 400mm PES

- insect screen panel 800mm PES

Door curtain color

- yellow RAL 1003

- orange RAL 2004

- red RAL 3000

- blue RAL 5002

- gentian blue RAL 5010

- green RAL 6005

- anthracite RAL 7016

- gray RAL 7035

- traffic gray B RAL 7043

- black RAL 9005

- white RAL 9010

- translucent

Cover options

Side column cover

- Stainless steel AISI 304

Drum and motor cover

- Stainless steel AISI 304

Drive and control options

- Acoustic signal 24VAC 80 dB at 1 meter ( horn when door is moving)

- Bottom beam including wireless detection device and photocells in side columns (in alternative to light curtains)

- Extra safety photocell

- UPS 1 Phase 230V

- Function Auto/Manual (includes switch on the control box)

- Function Manual closing with opening/closing using a common manual command (e.g. one single pull rope opens and closes the door)

- Function Manual closing with separate impulse (e.g. 2 buttons ▼▲)

- Function Two opening heights I/II with manual selection (switch included)

- Function Two opening heights I/II with automatic selection ( 2 different opening impulses)

- Function Interlocking / Airlock

Access and automation

- Spot on - touchless opening device

- Photocell open door

- Remote control

- Push button box with 1 opening button (arrow up)

- Push button with 1 EMERGENCY STOP button

- Push button with 1 EMERGENCY STOP with key

- Box with large black “mushroom” button for opening

- Key operated box open/close

- Push button stop

- Push button box 1-button

- Push buttons for opening and for closing

- Magnetic loop

- Radar incl. remote control

- Radar holder stainless steel – 950 mm

- Traffic lights

- Warning light Red

- Warning light Green

- Standard function flashing light

- Standard function flashing light with pre-warning before closing and opening

- Pull rope switch complete 5 m cord

- Pull switch bracket in stainless steel – L 3000 mm

- Wireless and batteryless pull rope switch 5 m cord

- Wireless and batteryless push button box