2021-01

ASSA ABLOY

HS9030GEM High speed door

Tender Specification Text

# ASSA ABLOY HS9030GEM High speed door

Product description

- The HS9030GEM is a high speed roll up door combined with an emergency exit for inside environments.

- An ingenious T-shaped cut, retained by zippers in the middle of the curtain allows the two symmetrical “flaps” to release an emergency exit, simply by pushing at the indicated release point (“Push here in case of an emergency”)

- Requires little space.

- Gravity drive system.

- Unique side column technology provides excellent seal on all 4 sides thereby reducing heat loss.

- The speed of opening is up to 2,0 m/s depending on size of door.

- The speed of closing is 0,5 m/s.

- Medium openings and high frequency of opening.

- The door design withstands overpressure or underpressure in specific areas.

- Photocells in side columns.

- Dimensions: Min: 1000 mm x 2200 mm, Max: 3500 mm x 3500 mm

- Wind load resistance: (EN 12424) Class 1 (300 Pa (N/m²))

- Resistance to water penetration: (EN 12425) Class 1 (30 Pa (N/m²))

- Air permeability (EN 12426) Class 0

- Thermal transmittance (EN 12428) 6,02 W/(m²K)

- Temperature working range: 5 °C to +40 °C

- Door curtain 900 g/m² colored PVC, material thickness: 0,8 mm

- Flexible door curtain without ridged parts.

- Flexible soft bottom beam including wireless detection device.

- Break-away and self reset function.

- The door curtain is available in 11 standard colors and translucent.

- Optional window (640 x 580), Material PVC, material thickness 2.0mm

- The motor with frequency converter guarantees smooth and reliable operation.

- Soft start/stop significantly prolongs motor lifespan.

- The operator is located inside the fabric drum, where it is protected from any dust and dirt.

- Frame and drum cover in glass fibre, aluminium color.

- Operator 220V - 240V 1-phase 0.75kW 16A

- Operator protection: IP65

- Control unit with display, impulse Up and Down buttons, an emergency stop mushroom button and a mechanical main switch.

- Control unit protection: IP65

- Lifetime expectations: 1.000.000 cycles

- Optional color for drum cover, side column cover and motor cover.

Options

Window and insect screen options

- vision panel 400mm PVC

- insect screen panel 400mm PES

Door curtain color

- yellow RAL 1003

- orange RAL 2004

- red RAL 3000

- blue RAL 5002

- gentian blue RAL 5010

- green RAL 6005

- anthracite RAL 7016

- gray RAL 7035

- traffic gray B RAL 7043

- black RAL 9005

- white RAL 9010

- translucent

Drive and control options

- Acoustic signal 24VAC 80 dB at 1 meter ( horn when door is moving)

- Extra safety photocell

- UPS 1 Phase 230V

- Function Auto/Manual (includes switch on the control box)

- Function Manual closing with opening/closing using a common manual command (e.g. one single pull rope opens and closes the door)

- Function Manual closing with separate impulse (e.g. 2 buttons ▼▲)

- Function Two opening heights I/II with manual selection (switch included)

- Function Two opening heights I/II with automatic selection ( 2 different opening impulses)

- Function Interlocking / Airlock

Access and automation

- Photocell open door

- Remote control

- Push button box with 1 opening button (arrow up)

- Push button with 1 EMERGENCY STOP button

- Push button with 1 EMERGENCY STOP with key

- Box with large black “mushroom” button for opening

- Key operated box open/close

- Push button stop

- Push button box 1-button

- Push buttons for opening and for closing

- Magnetic loop

- Radar incl. remote control

- Radar holder galvanized steel – 950 mm

- Radar holder painted steel – 950 mm

- Radar holder stainless steel – 950 mm

- Traffic lights

- Warning light Red

- Warning light Green

- Standard function flashing light

- Standard function flashing light with pre-warning before closing and opening

- Pull rope switch complete 5 m cord

- Pull switch bracket in galvanized steel – L 3000 mm

- Pull switch bracket in painted steel – L 3000 mm

- Pull switch bracket in stainless steel – L 3000 mm

- Wireless and batteryless pull rope switch 5 m cord

- Wireless and batteryless push button box