Aluminium High-performance door ASSA ABLOY RR3000

Description:

Aluminum High-performance door, opening in vertical direction, for interior and exterior use.

Self-supporting side frames of edged galvanized steel profiles. springs for curtain tension and counterbalance in the side frame.

Door curtain of ecru anodized double walled aluminium lamellas, hingeless connected by means of flexible heavy belts, wear-less to wind up without to contact each other, each single lamella can be replaced easily, pre-fitted on the top roll.

Wind resistance (EN12424): class 4, class 3 for doors DW > 3500 mm.

Leckage (EN12426): class 2.

Watertightness (EN12425): class 1.

Noise emission: in db (ISO 140-3): 25 dB (23 ohne MiWo).

Temperature range max.: -200 C bis + 600 C.

Drive unit: gear motor.

Protection: IP 55.

Control system:

MCC, with vector control technology and boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service

Control voltage 24 V DC.

Supply voltage:

3L/(N)PE 380/400/415/440/460/480V; 50/60 Hz

With transformator 220/230/500V; 50 Hz.

Safety devices:

- patented, contact-free pre-running safety photocell

- stationary safety photocell  integrated in side frame

- drop down protection by integrated balancing system

Manual activation: by releasing the brake at ground level. Partly opening by means of the pre-stressed tension springs.

Opening / closing speed: up to max. 2,2/0,7 m/s

Size max.:

(DW x DH) min: 600 x 3125 mm

(DW x DH) min: 5000 x 5000 mm

Make: ASSA ABLOY RR3000

Options

Covers:

- Top roll cover made of anodized aluminum

- Top roll cover powder coated

- Motor cover made of anodized aluminum

- Motor cover powder coated

Surface/material:

- Side frames with covers made of anodized aluminum

- Side frames coloured, RAL powder coated

- Aluminum lamellas coloured

Miscellaneous:

- Lamellas with windows

- Insulation

- Extended flexible seal

- Door as Laser Door

- Door as Machine Protection Door

- Contact edge

- Chain drive

Actuators