# ASSA ABLOY LH6080L Loadhouse

## Product description:

- The LH6080L Loadhouse is a housing that together with the Autodock loading platform forms a complete loading unit for attachment to the building.

- The complete loading unit is finished off with a sectional door in the outside wall of the building and a dock shelter on the front of the Loadhouse.

- Loading bay without thermal bridges. Full thermal separation between interior and exterior. Suitable for new builds and retrofitting.

- Dock leveler optionally as swingdock with swing lip or teledock with lip extension. Dock shelter optionally mechanical with PVC curtains or inflatable with bags in PVC. Sectional door optionally manual or electric.

- Modular design consisting of 2 wall elements and 1 roof element.

- Roof slope with an angle of 4° from rear to front

- The entire steel frame construction is hot-dipped galvanised.

- Self-supporting construction with proven statics.

- Steel construction designed in accordance with DIN 18800

- Assumed loads in accordance with DIN 1055 Section 4

- Basic wind load: 0,84 kN/m², Eurocode 3

- Basic snow load: 2,00 kN/m², Eurocode 3

- Accumulated snow load: 3,50 kN/m², Eurocode 3

## Dimensions (width / length):

- Nominal length: 2000, 2450, 3000 mm

•	Nominal width: 3300, 3500, 3600, 3750 mm

- Angular adjustment: 90°

## Dimensions (length / height):

Total Height > TH*

NL 2000	NL 2450	NL 3000

DH	950	3880	3910	3950

1000	3830	3860	3900

1050	3780	3810	3850

1100	3730	3760	3800

1150	3680	3710	3750

1200	3630	3660	3700

1250	3580	3610	3650

1300	3530	3560	3600

1350	3480	3510	3550

1400	3430	3460	3500

1450	3380	3410	3450

1500	3330	3360	3400

* Measurement only valid for insulated wall setup. For uninsulated wall setup TH is 60mm less. When the Loadhouse is supplied with rain pipe and gutter add 100mm.

## Cladding:

- Type HU - Uninsulated cladding with profiled galvanized sheeting, 0,6 mm profiled steel sheet.

- Type HI – Cladding with ISO panel thickness 40mm, steel sheet galvanized

- Type HX - Steel frame construction without cladding

Self-supporting design with reinforcements suitable for onsite cladding

## Angular adjustment:

- 90° to building (standard)

- 45° to building

- 135° to building

## Standard paints:

- Type HMU with profiled sheeting

RAL 1015 / RAL 3000 / RAL 5010 / RAL 6005 / RAL 6011 / RAL 6020 / RAL 7016 / RAL

7032 RAL 7035 / RAL 8004 / RAL 8011 / RAL 8012 / RAL 9002 / RAL 9006 / RAL 9007 /

RAL 9010

- Type HMI with ISO panel

RAL 1015 / RAL 7035 / RAL 9002 / RAL 9006 / RAL 9007 / RAL 9010

## Options:

- Drainpipe and gutter in galvanized finish

- Wall protection interior in veneer plywood up to a height of 1200 mm

- Roof section interior with anti-drip coating, only for HU design with profiled sheeting