Q2.1 - 2018

ASSA ABLOY

HS9010P High speed door

Tender Specification Text

# HS9010PX3 High speed door

Conform to directive (2014/34/EU):

■ Ex h IIC T6 EPL Gb

■ Ex h IIIC 85°C EPL Db

■ Ex db mc ia e IIC T4 EPL Gc

■ Ex tb mc tD IIIB 135°C EPL Dc

■ CE 0029 APRAGAZ 16ATEX 0150X/1

■ T ambient -10°C ~ +45°C

Product description

- The HS9010PX3 is designed for interior use in medium-sized heavy-duty operations.

- For ATEX environment category 3 G/D

- Unique door drive technology – a gear driven system consisting of a sprocket on the drive shaft that forces the lateral retaining straps up or down and contains no ballast in the door curtain or tension straps.

- Unique side column technology provides excellent seal on all 4 sides thereby reducing heat loss.

- All metal parts construction in stainless steel AISI304

- The speed of opening is up to 1.0 m/s

- The speed of closing is 1,0 m/s

- The door design withstands overpressure or under pressure in specific areas.

- Ex ia infrared Photocells in side columns 300mm from floor.

- Dimensions: Min: 1000 mm x 2000 mm, Max: 5500 mm x 5500 mm

- Wind load resistance: (EN 12424) Class 1 (300 Pa (N/m²)

- Water permeability (EN 12425) Class 3, (>50 Pa (N/m²)

- Air permeability (EN 12426) Class 1 (24 m³/m²/h at 50 Pa) for doors up to 3500x3500 mm

- Thermal transmittance (EN 12428) 6,02 W/(m²K)

- Temperature working range: -10 °C to +45 °C

- Black RAL9005 (or close to) Anti-static curtain

- Flexible door curtain without ridged parts.

- Flexible soft bottom beam including intrinsically safe wired detection device.

- Break-away and self reset function.

- The motor (Ex d Power 1,5 kW – IP rating: IP 65) with frequency converter guarantees smooth and reliable operation.

- Soft start/stop significantly prolongs motor lifespan.

- Operator 220V - 240V 1-phase 1.5kW 16A (option: 400V 3-phase 1.5kW 3 x 16A)

- Operator protection: IP65

- Control unit with display, impulse Up and Down buttons, an emergency stop mushroom button and a mechanical main switch.

- Control unit protection: IP65

Stainless Steel options

- Side column cover, stainless steel AISI304

- Motor cover, stainless steel AISI304

- Drum cover, stainless steel AISI304

Window and vision panel options

- Vision strip 350mm H max. 2 rows

Drive and control options

- Acoustic signal 24VAC 80 dB at 1 meter (horn when door is moving)

- Extra safety photocell

- UPS 1 Phase 230V

- Function Auto/Manual (includes switch on the control box)

- Function Manual closing with opening/closing using a common manual command (e.g. one single pull rope opens and closes the door)

- Function Manual closing with separate impulse (e.g. 2 buttons ▼▲)

- Function Two opening heights I/II with manual selection (switch included)

- Function Two opening heights I/II with automatic selection ( 2 different opening impulses)

- Function Interlocking / Airlock

Access and automation options

- Box with ATEX opening Push button

- Box with ATEX Emergency Stop button

- ASIS 304 support for box with 1 button

- ASIS 304 support for box with 2 buttons

- Pull switch ATEX complete with 5 m cord

- Pull switch bracket in ASIS 304 – L 3000 mm

- Bracket in ASIS 304 950mm

- Bracket in ASIS 304 500mm