High-performance door ASSA ABLOY RP2000 Machine Protection

Description:

High-performance door, opening in vertical direction, for use in material handling.

Side frames are self-supporting made of galvanized steel, balancing system integrated in the side frames.

Door curtain: Door curtain of anodized aluminum lamellas, hingeless connected by means of flexible heavy belts, wear-less to wind up without to contact each other, each single lamella can be replaced easily, pre-fitted on the top roll.

Opening / closing speed: max. 2.0/1.2 m/s

Wind resistance (EN 12424): class 3, for doors DW>3000 class 2

Drive unit: gear motor, pluggable electrical connections

Protection: IP 55

Control unit: MCC, with vector control technology and boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service

Control voltage: 24 V DC.

Supply voltage: 380 – 480 V (+/- 10%) 50/60 Hz

Safety devices:

- Pre-running photocell with integrated cable chain in the side frames,

- Drop down protection by integrated balancing system

- Safety limit switch: magnetic switch with reader performance level e according to EN ISO 13849-1 and SIL3, according to EN 62061, switch integrated in side frame.

Manual activation:

by releasing the brake at ground level. Partly opening by means of the pre-stressed tension springs.

Size max.: (DWxDH) 4000 x 3500 mm

Make: ASSA ABLOY RP2000

Options

Cover:

- Top roll cover, anodized aluminum

- Top roll cover, powder coated

- Motor cover galvanized steal

- Motoer cover, powder coated

- Cover for the backside of the top roll for self-supporting installation

Surface  / material:

- Side frames with covers made of anodized aluminum

- Side frames with colour coating

- Aluminum lamella with coloured coating

Curtain:

- Double walled slats  (laser protection) made of anodized aluminum

- Side frames with colour coating

- Slats with windows made of polycarbonate (250 x 40 mm)

Miscellaneous:

- Door line photocell in the side frames

- Extended flexible seal

- Supports for self-supporting installation

- Additional opening hight

- Actuator

- Additional safeties