High-performance door ASSA ABLOY RP300 Machine Protection

Description:

Flexible High-performance door, opening in vertical direction for use as a machine protection door.

Self-supporting side frames made of aluminium with integreated safety switches, all fittings galvanized.Front of side frame lifts of for generous access to safety limit switches and fixtures.

Door curtain made of RollTex® Plus – colour

- Blue 	(similar RAL 5002)

- Orange	(similar RAL 2009)

- Grey 	(similar RAL 7001).

Wind protection: through wind tabs on the door curtain

Wind resistance: class 1 (EN 12424) up to max. 11 m²

Bottom profile: anodized aluminium.

Drive unit: hollow shaft gear motor, rotable in steps of 45°

Protection: IP 55

Control system: Frequency converter control system MCC, based on Vector control and including boost function at start-up. Can be upgraded with a graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service.

Control voltage: 24 V DC

Supply voltage:

MCC: 3L(N)/380/400/415/440/460/480V; 50/60 Hz.

Transformer necessary: 3L(N)PE/220/230/500V; 50Hz.

Safety devices:

- Electrical safety contact edge

- door line photocell

- Drop down protection integrated in gear box.

- Safety switches: Magnetic switch with evaluation unit for performance level e, acc. to EN ISO 13849-1, and SIL3, acc. to EN 62061, integrated in side frame.

Manual activation: by means of a crank.

Opening / closing speed: max 2.4/1.3 m/s (depending on size)

Size max.:

ASSA ABLOY RP300

(DW min./max.) 750mm / 4000 mm

(DH min./max.)  750mm / 3500 mm

ASSA ABLOY RP300 Wide

(DW min./max.) 750 mm / 6000 mm

(DH min./max.)  750 mm / 5000 mm

ASSA ABLOY RP300 USD

(DW min./max.) 1000 mm / 5500 mm

(DH min./max.)  1000 mm / 3500 mm

Make: ASSA ABLOY RP300 / RP300 Wide / RP300 USD Machine Protection Door

Option

Model

- The ASSA ABLOY RP300 USD (Upside Down) machine protection door is designed so that you

can integrate it into your production area where bigger components and machinery must be arranged vertically.

Cover

- Top roll cover:made of anodized aluminum

- Motor cover: steel sheet, powdercoated (similar RAL 9006)

Door curtain (alternatively):

- RollTex Original in color:

- Blue 	(similar RAL 5002),

- Orange (similar RAL 2009)

- Grey 	(similar RAL 7001)

- NomaTex in color:

- Grey 	(similar RAL 7038)

- Blue 	(similar RAL 5005)

- Orange (similar RAL 2009)

- Yellow 	(similar 1023)

- Red 	(similar RAL 3000)

- Rolltex Plus and RollTex Original with window

- NomaTex with window or vision panels

- Window Anti-UV bronze coloured window, EN 1598 
(only for Curtain RollTex® and NomaTex®)

- Curtain NomaTex® flame-retardant without window or vision penal 
- Color Grey (similar RAL 7045)

- PVC with colored fabric reinforcemet strips:

- Grey	(similar RAL 7035)

- Blue 	(similar RAL 5010)

- Orange (similar RAL 2011)

- Yellow 	(similar 1021)

- Red 	(similar RAL 3002)

- White 	(similar RAL 9010)

- Black 	(similar RAL 9005)

- PVC with colored fabric reinforcement strips silicone-free

Alternative control unit:

Contactor control ACS 50, electronic control system with mini contactors including main switch and open-stop-close switch, control voltage 24 V DC (open/close), max. 1.0 / 1,0 m/s

Supply voltage: 3L(N)PE/230/380/400/415V; 50Hz, transformer for 440/480/500V necessary

Safeguarding the main closing edge:

Self-monitoring electrical safety edge

Safety switches (alternatively)

- 2 pcs Schmersal magnetic switches with control unit for safety class 4, Ple, SIL3, switches integrated in the side frames

- 2 pcs Telemecanique limit switches with control unit for safety class 3, PLd, SIL3, switches integrated in the side frames

- 1 pc Telemecanique limit switch on the drive side, no safety class

- 2 pcs Telemecanique limit switches, no safety class

Miscellaneous:

- Graphic display for MCC VectorControl unit

- Bottom plates in the side frames

- Braces for floor assembly

- Chain drive

- X-level extensions of the side frames from the lower closing edge to the floor

- Extended flexible seal

- Actuator

- Additional safety devices