2022-01

ASSA ABLOY

DL6130C Combidock

Tender Specification Text

# ASSA ABLOY DL6130C Combidock

Product description

Combidock extending bridge

- Special construction for efficient loading and offloading of all sizes of vehicle, from truck to small transporter

- Integrated frame with wall anchors (T-frames)

- Electro-hydraulic operation with two lifting cylinders

- Lip extension in aluminium, in three sections, each with an extender cylinder

- Control unit with selector for “Big Truck” and “City van” operation

- “Big Truck” operation: 
The lip is extended completely in one piece, total width 2000 or 2200 mm

- “City van” operation:
Only the central lip section is extended, width of central section 1000 mm
When positioned on the vehicle the bearing strength is reduced to approx. 100kg

- Automatic, free floating position

- Hydraulic emergency stop in any operating position

- Automatic mechanical locking in neutral position

- Lateral torsion flexibility

- Side plates (toe guards) with warning signs both above and below dock level

- Internal wiring, plug connection

- Power supply by others

- Control Unit Protection Class 950-series IP54

- The dock leveler meets standard demands of most loading operations and fully complies with rules and regulations of the European Standard EN 1398

Dimensions

Dimensions 500 mm lip

Vertical Working range 60kN mode**

| NL | LH | LP | A | B - inside LH* | B - down to LP | DH |
| --- | --- | --- | --- | --- | --- | --- |
| 3000 | 800 | 950 | 450 | 375 | 550 | 1200 |
| 3500 | 900 | 1150 | 560 | 275 | 650 | 1200 |
| 4000 | 900 | 1150 | 560 | 270 | 650 | 1200 |
| 4500 | 900 | 1165 | 560 | 260 | 650 | 1200 |

Vertical Working range 20kN mode**

| NL | LH | LP | A | B - inside LH* | B - down to LP | DH |
| --- | --- | --- | --- | --- | --- | --- |
| 3000 | 800 | 950 | 100 | 375 | 550 | 1200 |
| 3500 | 900 | 1135 | 100 | 275 | 650 | 1200 |
| 4000 | 900 | 1150 | 100 | 270 | 650 | 1200 |
| 4500 | 900 | 1165 | 100 | 260 | 650 | 1200 |

Dimensions 1000 mm lip

Vertical Working range 60kN mode**

| NL | LH | LP | A | B - inside LH* | B - down to LP | DH |
| --- | --- | --- | --- | --- | --- | --- |
| 3000 | 800 | 950 | 540 | 420 | 620 | 1200 |
| 3500 | 900 | 1135 | 620 | 300 | 700 | 1200 |
| 4000 | 900 | 1150 | 620 | 290 | 700 | 1200 |
| 4500 | 900 | 1165 | 620 | 280 | 700 | 1200 |

Vertical Working range 20kN mode**

| NL | LH | LP | A | B - inside LH* | B - down to LP | DH |
| --- | --- | --- | --- | --- | --- | --- |
| 3000 | 800 | 930 | 100 | 420 | 620 | 1200 |
| 3500 | 900 | 1135 | 100 | 300 | 700 | 1200 |
| 4000 | 900 | 1150 | 100 | 290 | 700 | 1200 |
| 4500 | 900 | 1165 | 100 | 280 | 700 | 1200 |

* When tail lift is in use 
Nominal width (NW) 2000mm (central lip section in “City van” operation: 1000 mm)
Nominal width (NW) 2200mm (central lip section in “City van” operation: 1200 mm)

** In accordance with the EN 1398 standard, the leveler must not be used beyond the permissible gradient range of ± 12.5% (around ± 7°). The limits may only be exceeded if the operator ensures that the danger of slipping has been eliminated (e.g. due to dry and clean surfaces).

Load capacity

- “Big Truck” operation 60kN

- “City van” operation 20kN

Surface

- Surface painted in standard color RAL 5010, thickness 80µm

Lip bearing

- 500 mm telescopic lip

Hydraulics

- Standard oil for temperature range -20°C - +60°C

Control unit

- Voltage 400V 3-phase, 230V 3-phase , cable harness length 18m

- Impulse auto button to put the leveler back in parking position

Options

Surface

- Surface painted in, RAL 3002, RAL 6005 or RAL 9005 thickness 80µm

- Surface painted with coat thickness 160µm

- Surface hot-dipped galvanised 80µm

Lip bearing

- 1000 mm telescopic lip

Frame

- W-Frame (welded to pre-cast frame)

Hydraulics

- Low temperature oil for temperature range -30°C - +60°C

- Bio-oil (environmental protection)

Energy & Ergonomics

- EPDM gap sealing

- Slip protection and noise reduction coating

Multifunctional docking control for direct control of dock leveler, dock shelter and door with just one control unit

- Operating dock leveler and door